Linux network commands:

- ifconfig command:

  - ifconfig --help, displays the parameters for this command.

  - ifconfig -a, lists all the network interfaces.

  - ifconfig eth0 10.0.3.2/24 up, activates the card "eth0" with ip 10.0.3.2 and mask 255.255.255.0.

  - 4. ifconfig eth0 hw ether 02: 32: 45: 11: 00: 11, changes the MAC address of the card eth0.

  - ifconfig eth0 down, disable the card eth0.

- route command:

  - route add default gw 10.0.3.1, insert into the routing table default gateway 10.0.3.1.

  - route del default, delete the default gateway.

  - route add -net 192.168.0.0/24 gw 192.168.0.1, insert into the routing table gateway 192.168.0.1 for the network 192.168.0.

  - route del -net 192.168.0.0/24, deletes the routing for the 192.168.0 network.

  - route -n, displays the routing table.

- arp command:

  - arp -n, displays the arp cache on the computer.

  - arp -d 10.0.2.1, delete the record ip 10.0.2.1 from the arp cache.

  - arp -s 10.0.2.1 00: 23: 45: 23: 45: A3, enter the ip-mac correlation "10.0.2.100: 23: 45: 23: 45: A3 "to the arp cache.

REFERENCES

[1] http://lartc.org/howto/ "Linux Advanced Routing & Traffic Control HOWTO"
[2] http://tldp.org/HOWTO/NET3-4-HOWTO.html "Linux Networking-HOWTO (Previously the Net-3 Howto)